Coding System

The B-series engine uses a pressurized, forced circulation cooling system with twin V-belt

driven centrifugal water pump and a wax pellet thermostat with a jiggle valve. A fan is solid mounted to the adapter in front of the water pump pulley to force-flow air across the radiator.

Coolant Flow Circuit:

l	Coolant is drawn from the bottom of the radiator by the water pump and pumped into

the cylinder block.

l The coolant is then channeled into the cylinder head and exits into the thermostat

housing at the front end of the cylinder head.

l Prior to operating temperature, the thermostat bypasses coolant from the water

outlet pipe and redirects the coolant into the bypass port of the thermostat housing. An Lshaped hose connects the bypass port outlet to the bypass inlet port of the water pump housing.

l Reaching operating temperature, the thermostat will begin to open. When the

thermostat opens, it will block off the bypass port of the thermostat housing and at the same time opens coolant flow into the water outlet pipe to allow coolant to reach the radiator where it is cooled. (Figure 2-39)

Water outlet pipe

- Cylinder	head

Figure 2-39

23