Valve Adjustment

1. Allow the engine to cool. Valve adjustment must be performed on a cold engine only.

2. Bring the engine up on number one compression stroke. Verify the position by

checking for clearance in between the valve stem tip and the rocker arm.

NOTE: If there is no clearance, then the number one cylinder is at the exhaust stroke.

Rotate the crankshaft an additional 360 4	In this position the following valves can be

adjusted.

6BG-Series

1			2			3			4			5			6 I		E	I		E	I		E	I		E	I		E	I		E

d	d	d	1111	11-w	d	d	1111	m-11	d	1111	1111

3.	Loosen the adjuster jam nut; insert a 0.40mm or 0.016 feeler gauge in-between the

rocker tips and the valve stems to adjust the valves.

4.	Rotate the crankshaft another 360° (the number one piston is now on its exhaust stroke) and adjust the remaining two valves

6BG-Series

1	2	3	4	5	6

I	E	I	E	I	E	I	E	I	E	I	E

NOTE: Proper adjustment is obtained when there is a slight drag against the feeler gauge.

26