Oil: Filter Service

Center Bolt Type: (Figure 3-l)

1.	Loosen the drain plug to drain the engine oil from the oil filter.

2.	Remove the center bolt and the filter body from the filter cover along with the filter element.

3. Discard the used filter element.

4.	Wipe the oil filter mounting surface with a clean rag to allow the new oil filter to seat properly.

5.	Install two O-rings A and B and check that they are correctly positioned.

6.	Place the new filter element inside the filter body, and reinstall with the center bolt.

l Torque the center bolt to 38 lb. ft.

l Torque the drain plug to 13 lb. ft. 7.	Check the engine oil level and replenish to the specified level if required.

8.	Start the engine and check for oil leakage and that the gasket is properly seated.

-- Paper elemenr

--- Gas ker w - v . . .-. Spring sfst

liilfi3- -.-....-*. spring

m -............ (-Jrjng	A

Figure 3-l

Spin-On Type: (Figure 3-2)

1.

2.

3.

4.

5.

6.

7.

8.

Loosen the used oil filter by turning it

counter-clockwise with the filter wrench. Discard the used oil filter.

Wipe the oil filter mounting surface with a clean rag to allow the new oil filter to seat properly.

Put a light coat of oil to the O-ring of the new oil filter.

Lightly turn the oil filter clockwise by

hand until the oil filter has seated to the mounting surface.

Use the filter wrench to further tighten the oil filter between % of a turn to one full turn.

Check the engine oil level and replenish to the specified level if required.

Start the engine and check for oil leakage from the oil filter.

Figure 3-2