Fuel Pump Strainer

A fuel pump strainer is screwed inside the fuel inlet banjo bolt at the bottom of the

transfer pump. Periodic inspection and cleaning is required, since it is the first stage of filtration of any incoming debris from the fuel tank (after any optionally equipped water separator). Replacement is required if any damage to the strainer screen is discovered during routine inspection. (Figure 3-3)

NOTE: The replacement strainer comes with a new banjo bolt and is sold as a complete

assemblv.

1.

2.

3. 4.

5.

6.

Figure 3-3

Remove the banjo nut from the bottom

of the feed pump using a 17mm wrench. (Figure 3-4)

Use a screwdriver to remove the strainer from the banjo bolt.

Wash the strainer in clean diesel fuel, or replace if damaged.

Reinstall strainer into banjo bolt. Reinstall banjo pipe, sealing washers, and banjo bolt to the transfer pump. Torque the banjo bolt to 41 N.m

(30 lb. ft.).

Figure 3-4

28