Main Fuel Filter

Small foreign particles which may damage the fuel injection pump or clog the injectors are

filtered by the main fuel filter. Depending on model configuration, the main fuel filter mav be a center-bolt type or a spin-on type. (Figure 3-5 and 3-6)

Y

Center-Bolt Type:

1.	Loosen the drain plug to drain the fuel from the fuel filter.

2.	Loosen the center bolt to remove the filter body, spring, washer, gasket, and filter element.

3. Discard the used element.

4.	Wash the filter body and other parts in clean diesel fuel.

5. Reassemble and install the parts in

reverse order using new filter.

6.	Prime the fuel system and check for any fuel leakage.

Figure 3-5: Center-Bolt Type

Figure 3-6: Spin-On Type

Spin-On

1.

2.

3.

4. 5.

6.

7.

8.

Type:

Loosen the fuel filter by turning it

counterclockwise.

Discard the used filter.

Wipe the fuel filter mounting surface with a clean rag.

Apply a light coat of engine oil to the Oring.

Fill the filter canister with clean diesel fuel.

Lightly turn the fuel filter clockwise by

hand until the fuel filter has seated to the mounting surface.

Use the filter wrench to tighten the fuel filter an additional 2/3 turn.

Prime the fuel system and check for any fuel leakage.

NOTE: Check the overflow valve banjo

bolt for clogging and the check ball inside for insuficient seating. The over flow valve is the check ball banjo bolt attaching the fuel leak-offpipe to the top of the fuel filter mounting adapter.

29