Spill Port Timing

Spill-port timing is a precise method of measuring the initial injection of fuel by the pump

and the duration of injection.	It is a standard procedure used by Isuzu when timing Zexel

pumps. This procedure is a very precise method of timing fuel because it shuts off

fuel to the high-nressure chambers relative to the position of the niston.

.L,

u I

1. Turn the crank to align the crankshaft

pulley to TDC for the number one cylinder.

2. Remove the camshaft gear inspection

plate and verify the alignment of the

stationary pointer of the gear case and the #l TDC notch mark on the camshaft gear. (Figure 3-7) The engine is now at TDC for the number one cylinder.

NOTE: If the pointer and the mark does not line up, turn the crank an additional

360 Oand repeat this step.

Figure 3-7

3. Turn the engine counter-clockwise

approximately 30°.

4. Disconnect the injection pipe from the

#l nozzle and delivery valve holder.

5.	Loosen the center bolt of the “wedge

clamp” that secures the #l and #2 delivery valve holders, and remove the clamp. (Figure 3-8)

Figure 3-8

6.	Loosen the delivery valve holder and

remove it from number one high pressure chamber on the pump. (Figure 3-9)

Figure 3-9