Spill Port Timing (continued)

9.	Hold the fuel control lever at the wide-open position.

10. Slowly rotate the crankshaft pulley clockwise while at the same time pumping the

priming pump. Fuel will flow out of the delivery holder during this time. Continue until fuel stops flowing from the delivery holder. The crank position where the fuel stops flowing is the beginning point of injection for the number one cylinder.

11. Observe the degree reading at the crank pulley and compare it with the “INITIAL

INJECTION TIMING” specification in the Engine Certification Label. If an adjustment is required, refer to “Injection Pump Timing Adjustment” portion of this section (next page).

NOTE: Injection pump timing may vary among identical engine series due to the

various versions of the engine series.	The build specification code applicable to that

engine can be found in the Engine Production Spec Label on the engine.

12.Remove the delivery valve holder from

the number one high pressure chamber and reinstall the spring and valve stop. (Figure 3-13)

NOTE: Inspect the O-ring of the

delivery valve holder for any damage before reinstallation. A damaged O-ring will lead to leakage of fuel during engine operation.

Figure 343

13.Reinstall the delivery valve holder to the injection pump and tighten the delivery valve

holder to the specified torque range of 29-33 lb.ft.

14.Reinstall the “wedge clamp” to secure the #l and #2 delivery valve holders and tighten

the center bolt of the clamp to the torque range of 6-9 lb.ft.

15.Reinstall the injection pipe and tighten the injection pipe nuts to the specified torque

range of 21-24 lb. ft.

32