Injection Pump Timing Adjustment

If the spill port timing procedure reveals that the injection timing is off a few degrees from

specification, while the timing gears are timed correctly, fine adjustments may be made to bring the injection timing to specification., Any adjustments to the injection pump timing, must be made during the spill port timing procedure. Adjustment location differs between direct gear driven pumps (4BG-series) and coupling driven pumps (GBG-series).

The 4BG-seties engines are equipped with

timing gear case mounted injection pumps with gears that are driven by the central idler gear. The pump body is mounted to a flange which bolts directly to the rear facing side of the timing gear case.

(Figure 3-14)

Figure 3-14

1.	Fine adjustment to the pump timing is

made by loosening the injection pump body fixing nuts (4 nuts at the flange and 1 nut at the rear support bracket) and rotating the pump*

l Counter-clockwise rotation (from the

front) will advance injection pump timing. l Clockwise rotation (from the front) will retard injection pump timing.

2.	Retighten the pump body fixing nuts

then recheck the pump’s timing using the spill port method.

The GBG-series engines are equipped with

an adjustable coupling of the pump drive to allow the pump timing adjustments. Generally, this setting is set at the center mark and allows the pump timing to be advanced or retarded up to 2O in either direction. (Figure 3-15)

Figure 3-15

1.	Fine adjustment to the pump timing is

made by loosening the two coupling bolts and rotating the pump drive coupling.

l Counter-clockwise rotation (from the

front) will advance injection pump timing.

l Clockwise rotation (from the front) will

retard injection pump timing.

2. Retighten the pump’s coupling bolts

then recheck the pump’s timing using the spill-port method.

33