Ejector Service

Figure 3-16

The following procedures are service

recommendations from Zexel:

1. Thoroughly clean all carbon residue and

carbon build-up on the surface of the nozzle using Zexel cleaning kit #1057790010.

(Figure 3-16)

2.	Soak all parts other than the nozzle in cleaning oil.

3. Use a soft wire brush to clean

excessively dirty parts.

IMPORTANT: Do not use metal or

abrasive cleaning media to clean the nozzle holder. Their abrasive nature will leave scars on the ground surface and may cause fuel leaks.

4. Remove the needle valve from the

nozzle. Clean the seat’s surface and shaft section using the piece of hardwood from the cleaning kit or a clean soft cloth dipped in oil.

(Figure 3-17)

5. Rewash the injector components to

remove any final debris.

6. Perform a needle slide test to ensure

that there is no obstruction that could cause a fuel leak. (Figure 3-18)

Figure 3-M

34