injector Pop-Off Test

WARNING: Fluid from the nozzle will

spray out under great pressure. It can easily puncture a persons skin. Keep your hands away from the nozzle tip at all times. For operator protection, it is recommended that a clear container be used to shield the tip of the nozzle during testing.

This procedure is used to check injection

starting (nozzle opening) pressure, spray pattern, chatter and leakage.

1. Mount the nozzle and holder assembly

to the tester and bleed the system of air.

2. Open the pressure gauge valve and

pump the lever at a rate of one stroke per second to check the opening pressure. When the pressure gauge pointer moves as shown below, read PO (pop-off) value. (Figure 3-24 and 3-25)

IMPORTANT: If the nozzle opening

pressure is not steady or falls off quickly9 dirt may be caught between the needle and holder assembly or the nozzle may be incorrectly assembled.

3.	If the nozzle “pop-off’ pressure does not

meet factory settings, adjust the injector pressure by turning the nozzle adjusting screw. (Figure 3-26)

NOTE: Each ?4 turn of the screw changes the “pop-off pressure by approximately 200 psi. Re-clean or replace any nozzles that cannot be brought to specification.

PM,,.‘X PainSac mQverTze?>$

/

‘..	.J..f \

‘k.	..J/.’

Figure 3-24

Figure 3-25

Figure 3-26

36