Injector Leak Test

The leak test should be conducted immediately following the nozzle opening pressure

adjustment test.

a Wipe the nozzle with a clean shop towel.

3 Decrease the tester pressure about 20

kg/cm2 less than the pop-off pressure and maintain the position.

+ There should be no fuel discharge from

the nozzle for at least 10 seconds. (Figure 3-27)

a Re-clean or replace any nozzle that

doesn’t meet the criteria.

Passable	Not passable

Figure 3-27

37