Glow Plug Inspection

The following procedure tests the resistance value of the.glow plug.	The test can be

performed on or off the engine.

NOTE: Individual glow plug resistance value should be SO.9 ohms.

On engine service:

CI	Remove the buss bar from the glow plug.

o	Attach the red lead to the top of the glow plug. o	Attach the black lead to a good ground source (verify the ground integrity by testing it with the DVOM. Reading should not be > 0.5 ohms).

Off engine service:

o	Disconnect and remove the glow plug.

o	Place DVOM leads across the glow plug (one lead at each end) to obtain resistance reading. (Figure 3-28)

Figure 3-28

38