Coding System Service Information

Ph levels between the 7.5-8.5 range on a Litmus paper test. Papers are available from

most tool suppliers.

Maintain a solution concentration ratio of 50/50. Use a commercially available anti

freeze tester.

Use only soft demineralized water, since the hard waters tends to neutralize the

corrosion inhibitors in the antifreeze.

Do not add any additional water pump lubricants to the cooling system.

Check the integrity of the radiator cap and cooling system cap pressure requirements.

NOTE: The water pump can be completely rebuilt.

39