Engine Maintenance Specifications

Engine	Oil	Capacity	0	4BG-series: 13.2 Liters	4BG-series: 3.5	Gal

GBG-series: 21.5 Liters

Engine Oil PressureGBG-series: 5.68 gal 4.078 - 4.99 Kg/cm2		58-71 lbs

Compression Pressure 0

I Valve Adiustment

31 Kg/cm2

441 psi

I	0.4 mm	I	0.016	I

Coolant Capacity 0

Injection Pressure @

4BG-series:	8.5	Liters	4BG-series:	2.25	gal

GBG-series:	12	Liters	GBG-series:	3.2	gal

4BGl:	150	4BGl:	2134	psi

4BGlT:	185	Kg/cm2	4BGlT:	2630	psi

GBG-series:	185	Kg/cm2	GBG-series:	2630	nsi

~~ 16°-200 BTDC

4BG-series:	8 	12	mm	4BG:	0.314 - 0.472	in

GBG-series:	10 	15	mm	6BG:	0.393-0.590	in

These specifications vary depending upon the type of equipment in which the engine

is being installed. Only use API class CC or CD.

Repair engine as necessary if compression pressure is 5 26.0 Kg/cm2 or 370 psi

Use only an ethylene glycol based anti-freeze/ water mixture. A 50/50 concentration

ratio provides maximum protection to -37OC or -34OF. Never exceed a GO/40 concentration ratio, which provides protection to = -50°C or -58OF.

Opening pressure; note that this spec depends upon the type of injector.

Specification will vary depending upon the engine specification.

Belt deflection is measured at the point midway between the water pump pulley

and alternator pulley with 10 kg (22 lbs.) of depressing force.

40