Turbocharger Boost Pressure Diagnosis

Listed below are suggested checks for determining the cause of lowered turbo boost

pressure (in the sequence they should be performed). To obtain maximum boost pressure, the engine must be operated at rated RPM under a full load condition. Boost pressure is measured with a mercury manometer. A pressure gauge may be substituted.

One of the following checks will uncover the reason for low boost pressure:

1.

2.

3.

4.

5.

6.

7. 8a

9.

10.

Check the throttle linkage for travel to full fuel position.

Measure the maximum no load engine RPM. Adjust if necessary.

Inspect the manifolds and turbo for cracks, loosened mounting bolts or leaking

gaskets.

Check the intake and exhaust systems for restriction, i.e., dirty air cleaner, collapsed

hose or crushed exhaust pipe.

Check the

fuel filter,

Check the

fuel system, i.e., air in the fuel, dirty fuel inlet screen (rock stopper), dirty

contaminated fuel or reduce fuel delivery to the injection pump.

valve clearance adjustment.

Check the	injection nozzles, i.e., popping pressure, spray pattern or leakage.

While the injection nozzles are removed, check the compression pressure.

With the turbo mounted to the manifold inspect the following:

It is necessary to remove the intake and exhaust piping from the turbo.

a) Check the compressor wheel for damage.

b) Check the turbine side for heavy carbon deposits or damage.

c) Measure the wheel shaft end play and bearing clearance.

Have the injection pump tested by an authorized Zexel dealer for proper

calibration.

55