Lubrication System (continued)

Oil Pump:

The oil pump is a gear type pump that uses an intermediate drive shaft driven off of the

camshaft gear. (Figure 2-34)

Figure 2-34

NOTE: Always remove the pump’s pressure relief valve during any rebuild.	Look for a cocked pressure spring, burred relief valve or burred pump housing that would cause the

pressure valve to fail. Also inspect the clearance in-between the gears and pump housing.

IMPORTANT: In order to remove the camshaft, the oil pump drive shaft must first be

removed.	20