## GENSETS

## PC-12-13

Date 8-12-03

SUBJECT:	          Incorrect location of  brake control wire on 6 way plug for trailers

MODELS AFFECTED:        G20 through G575

SYMPTOMS:        Brakes lock up even though tow vehicles brakes are not applied

CAUSE:                Brake control wire located on wrong pin of six way plug

REMEDY:           Installing brake control wire to correct pin location

ACTION:	      Relocate brake control wire to the correct pin shown in the drawing

below

# Trailer

## Trailer Connector 6-way Plug Wiring

## 6 Way Plug

Running Lights - Brown

Trailer Brakes – Blue/Black

Ground - White

End View

Right Turn - Green

Left Turn - Yellow

Accessory - Red